Trouble Shooting of error message of V4 software:

| Error Message | Possible Reason | Solution |
| --- | --- | --- |
| No Data in Device. | Log memory error | Perform a reset in V4 SW -> Config GPS. |
|  | Not log valid GPS fix data | V4 software will rule out the GPS not fix data, so make sure to log valid GPS data under open sky environment. |
| Failed to read data from GPS module! | Invalid operation | - Unplug and plug in the USB cable. - Switch off and on the device |
|  | Com port detection error | - Manually select connection com port at Config GPS and check connection com port at Windows Device Manager -> Com&LPT -> GPS USB…..or CP210X…… - Change another USB port for connection |
|  | Driver not installed properly | Reinstall the driver. For Q1000P, Q1000X, Q1200, Q1200Ultra, Q2000: Please remove the USB driver from Control Panel -> Remove Program -> Silicon Laboratories CP210x USB to UART Bridge……. And reinstall the driver. For Q1300, Q1300S: Please remove the USB driver when the device is connected to PC and find “GPS USB Serial Interface Driver” at Device Manager and right click on it and press Uninstall. Then the confirmation dialog will appear and please select “Also delete device driver software.” And reinstall the driver. |
| Switch is not turned on or incompatible GPS device! | not turn on the device | - Turn on the device. - Unplug and plug in the USB cable. |
|  | Incompatible GPS device | Check the compatibility of your Qstarz GPS module Travel Recorder PC Utility is compatible with following Qstarz GPS: Q1000P, Q1000X, Q1200, Q1200Ultra, Q1300, Q1300S, Q2000 |
| Failed to connect GPS Module! | GPS not correctly connected | Follow the standard operation procedure in user’s manual for making the connection |
|  | Com port detection error | - Manually select connection com port at Config GPS and check connection com port at Windows Device Manager -> Com&LPT -> GPS USB…..or CP210X…… - Change another USB port for connection |
|  | Driver not installed properly | Reinstall the driver. For Q1000P, Q1000X, Q1200, Q1200Ultra, Q2000: Please remove the USB driver from Control Panel -> Remove Program -> Silicon Laboratories CP210x USB to UART Bridge……. And reinstall the driver. For Q1300, Q1300S: Please remove the USB driver when the device is connected to PC and find “GPS USB Serial Interface Driver” at Device Manager and right click on it and press Uninstall. Then the confirmation dialog will appear and please select “Also delete device driver software.” And reinstall the driver. |
|  | GPS USB connection chip is faulty | - “CP210x…” or “GPS USB...” does not appear at Windows Device Manager -> Com&LPT when the device connected with PC. - Request a RMA to the store you bought from. |
| Failed to write data into GPS Module! | Invalid operation | - Unplug and plug in the USB cable. - Switch off and on the device - Try to read log again and don’t unplug during read log process |

*Every time when you feel the device works abnormally, please perform both hard and soft reset to let device back to normal status.

**If above method can not solve your problem, the device may be defective. Please request a RMA to the store you bought from.